MAINTENANCE STANDARD OPERATING PROCEDURE

176th Engineer Brigade (TXARNG)

Administrative Data

| Field | Value |
| --- | --- |
| HIGHER Headquarters | TEXAS MILITARY DEPARTMENT |
| HIGHER Headquarters L2 | TEXAS ARMY NATIONAL GUARD (TXARNG) |
| ADDress | P.O. BOX 5218 |
| ADDress L2 | AUSTIN, TX 78763-5218 |
| Document Title | 176TH EN BDE GROUND MAINTENANCE STANDARD OPERATING PROCEDURES |
| Document Type | STANDARD OPERATING PROCEDURE |
| Effective Date | [autofill |
| Supersedes | [Previous SOP Date or "N/A"] |
| Review Date | [Annual Review Date] |
| Approval Authority | Zebediah E. Miller, Colonel] |
| Distbution | A |
| Proponent | 176th Eng BDE S4 |

Summary: This SOP establishes standards, policies, and procedures for the execution of maintenance operations within the 176th EN BDE. It provides guidance for maintenance management, operator training, dispatching, safety, and documentation in accordance with AR 750-1, DA PAM 750-8, AR 385-10, and applicable TXARNG policies. When used in conjunction with applicable regulations and technical manuals, this SOP establishes the maintenance program for the 176th EN BDE.

Commanders Review: Maintenance Responsibilities Chapter 4; TAMMS and Records Chapter 5; GCSS-Army Chapter 6; CMDP Chapter 7; Dispatching Chapter 8; Operator Training Chapter 9; Safety Chapter 10

Applicability: This SOP applies to all units and personnel assigned or attached to the 176th EN BDE who operate, maintain, supervise, or manage equipment. Personnel operating vehicles or equipment must meet all training, qualification, and licensing requirements in accordance with applicable Army regulations and unit SOPs.

Supplementation: Supplementation of this SOP is authorized at the unit level. Subordinate unit SOPs will not conflict with this SOP or higher-level regulations and will be approved by the unit commander.

Suggested Improvements: The proponent for this SOP is [the 176th EN BDE S4]. Users are encouraged to submit recommended changes or improvements through their chain of command to the Brigade S4.

Distribution. A

1. PURPOSE

This SOP establishes procedures for the execution of maintenance operations within 176th Engineer Brigade (TXARNG) in accordance with AR 750-1, DA Pam 750-1, and applicable Technical Manuals.

2. APPLICABILITY

This SOP applies to all personnel assigned or attached to 176th Engineer Brigade (TXARNG) who perform, supervise, or manage maintenance operations.

3. REFERENCES

• AR 750-1, The Army Maintenance Management System (TAMMS)

• DA Pam 750-1, Commanders' Maintenance Handbook

• DA Pam 750-8, The Army Maintenance Management System (TAMMS) Users Manual

• Applicable Technical Manuals (TMs) for assigned equipment

4. RESPONSIBILITIES

4.1 Commander

• Ensures maintenance program compliance with regulatory requirements

• Appoints maintenance personnel in writing

• Reviews maintenance status at least monthly

4.2 Maintenance Officer

• Supervises all maintenance operations

• Ensures maintenance procedures comply with regulations

• Reviews and validates maintenance requests

4.3 Motor Sergeant / Maintenance NCO

• Supervises daily maintenance operations

• Ensures equipment is maintained to standard

• Manages maintenance personnel and work assignments

4.4 Operators

• Perform PMCS before, during, and after equipment operation

• Report deficiencies to supervisor immediately

• Maintain operator-level equipment records

5. PROCEDURES

5.1 Preventive Maintenance Checks and Services (PMCS)

a. Operators will perform PMCS IAW applicable TM -10 and -20 series manuals

b. Before operations: Complete before PMCS and document faults

c. During operations: Monitor equipment for abnormalities

d. After operations: Complete after PMCS and document findings

e. Weekly/Monthly: Complete scheduled services as required

5.2 Fault Reporting

a. Operators report faults immediately to supervisor

b. Supervisor evaluates fault and determines repair level

c. Document faults on DA Form 5988-E or in GCSS-Army

d. Non-mission capable faults reported to commander within 24 hours

5.3 Maintenance Requests

a. Submit maintenance requests through GCSS-Army

b. Include accurate fault description and symptoms

c. Supervisor validates and prioritizes requests

d. Track work orders to completion

5.4 Scheduled Services

a. Conduct scheduled services IAW TM requirements

b. Document services in GCSS-Army

c. Order parts in advance of scheduled services

d. Coordinate shop time for major services

---

6. RECORDS AND REPORTS

• DA Form 5988-E (Equipment Maintenance and Inspection Worksheet)

• GCSS-Army maintenance records

• DA Form 2408 series as required

• Monthly maintenance status reports

7. TRAINING

All personnel will receive training on:

• PMCS procedures for assigned equipment

• GCSS-Army maintenance transactions

• Fault reporting procedures

• Safety requirements for maintenance operations

8. SAFETY

• Use required PPE during all maintenance operations

• Follow TM safety precautions

• Report unsafe conditions immediately

• Lock out/tag out procedures for equipment under repair

APPROVAL:

[Image not found: ../memos/img/commander-signature.png]

[LAST NAME, FIRST NAME MI.]

[RANK, BRANCH, COMPONENT]

Commanding

CMDP Reference: 10(1)-6, 10(1)-7, 10(1)-8