BATTERY MAINTENANCE STANDARD OPERATING PROCEDURE

[UNIT NAME]

## Administrative Data

| Field | Value |
| --- | --- |
| Effective Date | [DD MONTH YYYY] |
| Supersedes | [Previous SOP Date or "N/A"] |
| Review Date | [Annual Review Date] |
| Approval Authority | [Commander Name, Rank] |

1. PURPOSE

This SOP establishes procedures for battery maintenance, testing, storage, and disposal within [UNIT NAME] in accordance with TB 43-0134 and DA Pam 750-1.

2. APPLICABILITY

This SOP applies to all personnel assigned or attached to [UNIT NAME] who handle, test, charge, store, or turn in batteries.

3. REFERENCES

• TB 43-0134, Battery Disposition and Disposal

• DA Pam 750-1, Commanders' Maintenance Handbook

• TM 9-6140-200-14, Lead-Acid Storage Batteries

• Installation HAZMAT/Environmental SOP

4. RESPONSIBILITIES

4.1 Commander

• Establishes battery maintenance program

• Appoints Battery Maintenance Operator in writing

• Ensures resources for battery maintenance

4.2 Battery Maintenance Operator

• Manages battery testing and maintenance

• Maintains battery records

• Coordinates battery turn-in

4.3 Operators

• Report battery deficiencies

• Perform operator-level battery care

• Use proper PPE when handling batteries

5. PROCEDURES

5.1 Battery Testing

a. Test batteries IAW TB 43-0134 using approved equipment

b. Test batteries at least quarterly or when deficiency suspected

c. Document test results

d. Condemn batteries that fail testing criteria

5.2 Battery Charging

a. Location:

• Charge batteries only in designated, ventilated areas

• Ensure adequate ventilation to prevent hydrogen buildup

• No smoking or open flames within 50 feet

b. Procedures:

• Inspect battery before charging

• Verify electrolyte level before charging

• Connect charger properly (positive to positive)

• Monitor charging progress

• Allow battery to cool before installation

5.3 Battery Storage

a. Serviceable Batteries:

• Store in cool, dry, ventilated area

• Store on wooden pallets or approved racks (not concrete)

• Maintain batteries in charged condition

• Inspect stored batteries monthly

b. Unserviceable Batteries:

• Store separately from serviceable batteries

• Mark clearly as "UNSERVICEABLE"

• Prepare for turn-in promptly

• Store upright to prevent leakage

5.4 Battery Handling Safety

a. Required PPE:

• Acid-resistant apron

• Face shield or safety goggles

• Acid-resistant gloves

• Safety shoes

b. Precautions:

• Never smoke near batteries

• Remove jewelry before handling

• Lift batteries properly using handles/carriers

• Never place tools or metal objects on batteries

5.5 Battery Turn-In

a. Complete turn-in documentation

b. Coordinate with supply for DRMO turn-in

c. Transport batteries upright and secured

d. Maintain turn-in records

5.6 Spill Response

a. Neutralize acid spills with baking soda

b. Wear required PPE during cleanup

c. Dispose of cleanup materials as HAZMAT

d. Report spills IAW Environmental SOP

---

6. BATTERY MAINTENANCE AREA REQUIREMENTS

• Emergency eyewash station within 25 feet

• Spill kit with neutralizing agent

• Fire extinguisher (Class B/C)

• Adequate ventilation

• Posted safety procedures

• Required PPE available

• Battery testing equipment

7. RECORDS AND REPORTS

• Battery test records

• Turn-in documentation

• Inspection checklists

• Training records

8. TRAINING

Battery Maintenance Operators will receive:

• TB 43-0134 familiarization

• Battery testing equipment operation

• Safety and PPE requirements

• Spill response procedures

APPROVAL:

LAST NAME, FIRST NAME MI.

RANK, BRANCH, COMPONENT

Commanding

CMDP Reference: 10(14)-3